Boat Monitor — Project 2

Power · Location · Bilge Monitoring

System Design & Architecture — v7.0  (Final — Pico Integration Ready)

May 2026

# 1. System Overview

This document covers the second Pico W monitoring project for the boat — separate from the ballast flow meter monitor (Project 1). It addresses four operational states with different power, connectivity, and data-capture needs, plus continuous bilge monitoring.

Key architecture decisions (locked):

- Single Raspberry Pi Pico W — handles all modes on one device

- Cellular: Waveshare SIM7600X 4G HAT (SIM7600NA-H) — confirmed working on T-Mobile LTE Band 4 in Seattle

- GPS: Built into SIM7600X HAT — tested via AT+CGPSINFO

- SIM: T-Mobile — disable SIM protection before deploying (confirmed registration blocker)

- Power bank: Voltaic V50 (13,400mAh, Always On mode). Charges when boat battery switch is ON.

- SIM7600X power control: AT+CPWROFF to minimize module power when not needed. No MOSFET required — use software power management. PWR–3V3 jumper = auto-boot on power-up.

- Winter storage: Pico in deep sleep (~0.5mA). RTC wakes every hour. Night: bilge check only. Day: read sensors, upload, sleep. SIM7600X powered down via AT+CPWROFF between transmissions.

- In-season: Pico always-on (~40mA). V50 lasts ~14 days per charge.

- UART pins: GP4 (TX) → SIM7600X RXD, GP5 (RX) → SIM7600X TXD — avoids USB serial debug conflict on GP0/GP1

- I2C pins: GP8 (SDA) and GP9 (SCL) — moved off GP4/GP5 to free UART1

- Bilge wiring: 12V bilge signal → optocoupler → output wired to BOTH a GPIO pin AND the Pico RUN pin. Same wiring works in all seasons.

- Hardware watchdog enabled in all always-on modes — auto-reboots if code hangs.

- End-of-day alert: battery switch ON for > 30 min → Pushover alert.

- WiFi primary, cellular fallback for all data uploads.

- Config split: WiFi credentials, Pushover key, pin assignments → config.py (BLE readable/writable). Operational thresholds → Google Sheet Config tab.

| Mode | Trigger Condition | Primary Goal | Connectivity | Pico Sleep? |
| --- | --- | --- | --- | --- |
| 1. Winter Storage | Battery switch OFF, solar connected | Log solar charging & battery voltage hourly | WiFi primary, Cell backup | YES — RTC wake every hour |
| 2. Underway | Battery line >13.5V (alternator charging) | GPS track every 30 sec | Cellular | No — watchdog enabled |
| 3. At Anchor/Dock | 12V switch line ON, voltage 12–13.5V | Location + anchor watch | Cellular | No — watchdog enabled |
| 4. End-of-Day Alert | Switch ON, boat stationary >30 min | Alert owner, log, wait for switch-off | Cellular / WiFi | No — until switch OFF |
| Bilge (always) | Any — runs across all modes | Log pump runs, float switch events | WiFi / Cell | Winter: RUN pin wake. In-season: GPIO IRQ. |

# 2. Hardware

## 2.1 Hardware Inventory

| Component | Role | Interface | Status |
| --- | --- | --- | --- |
| Voltaic V50 USB Battery Pack | 13,400mAh, Always On mode. Powers Pico independently of boat battery switch. Charges when battery switch is ON via PlusRoc buck converter. | USB-A → Pico via TPS2113A | Have |
| Raspberry Pi Pico W | Main controller — all modes, all sensors | — | Have (×1) |
| Waveshare SIM7600X 4G HAT (SIM7600NA-H) | PRIMARY cellular + GPS. Confirmed working on T-Mobile LTE Band 4 in Seattle. North America variant. Connected to Pico via UART breakout (not GPIO header). Powered separately from 5V. | UART (GP0/GP1) | Have — CONFIRMED ✓ |
| INA260 (×2) | #1 in series on house solar charger output — measures house solar current AND house battery voltage. #2 same on engine solar charger — measures engine solar current AND engine battery voltage. Engine alternator detected when INA260 #2 reads >13.5V. | I2C (GP4/GP5) | Have |
| INA219 | Monitors Voltaic V50 USB battery bank — voltage and current. Tracks bank health over winter. NOT used for main boat batteries. | I2C (GP4/GP5) | Have |
| PC817 Optocoupler board (8 channels) | Converts all 12V boat signals → 3.3V GPIO-safe. See Section 2.3 for channel assignments. | GPIO (digital IN) | Have (×2 boards) |
| Pololu TPS2113A Power Mux (#2596) | Autoswitches between V50 USB bank (IN1) and 12V switch line via PlusRoc (IN2). Seamless failover with no power interruption to Pico. | OUT → Pico VSYS | Have |
| PlusRoc 12V→5V Buck Converter (waterproof) | Steps down boat 12V battery switch line to 5V. Output splits: Branch 1 → TPS2113A IN2. Branch 2 → V50 USB-C charging input. | Wired inline | Have |
| SRD-05VDC-SL-C Relay (×2) | Reserved for future use. | GPIO (digital OUT) | Have |

## 2.2 SIM7600X HAT — Confirmed Setup & Jumper Configuration

The SIM7600X is a full Raspberry Pi HAT. Bench testing confirmed working via USB. Pico connection is via the UART breakout header. Three jumpers on the board must be set correctly:

| Jumper | Set To | Meaning | Notes |
| --- | --- | --- | --- |
| UART | Position B | Control SIM7600 via UART breakout pins | NOT position A (USB debug) or C (USB control) |
| PWR | PWR–3V3 (default) | Module auto-boots when 5V applied | No PWRKEY pulse needed from Pico |
| VCCIO | 3.3V | UART logic level = 3.3V | Matches Pico GPIO logic |
| Flight | NC (default) | No flight mode control | Leave unconnected |

### UART breakout wiring (Pico ↔ SIM7600X):

- Pico GP4 (UART1 TX) → SIM7600X RXD breakout pin

- Pico GP5 (UART1 RX) → SIM7600X TXD breakout pin

- Pico GND → SIM7600X GND breakout pin

- GP4/GP5 use UART1 — avoids conflict with USB serial debug on GP0/GP1 (UART0)

### Power wiring:

- SIM7600X powered from same 5V rail as Pico — PlusRoc buck output or V50 USB port

- Do NOT power SIM7600X from Pico 3.3V or VSYS — LTE transmit peaks at 1–2A will brown out the Pico

- Both Pico and SIM7600X share common GND

- No MOSFET or relay needed — use AT+CPWROFF to minimize module power consumption between transmissions

### Optional control pins (connect to Pico GPIO):

- SIM7600X RESET → Pico GP14 — allows Pico to hard-reset module if it hangs at sea

### Antenna connections:

- LTE antenna → MAIN U.FL connector

- GPS antenna → GNSS SMA connector

- AUX U.FL — leave unconnected (diversity antenna, optional)

### Confirmed AT commands:

- AT → OK

- AT+CEREG? → +CEREG: 1,1  (registered T-Mobile LTE)

- AT+CPSI? → LTE,Online,310-260,EUTRAN-BAND4  (T-Mobile Band 4 Seattle)

- AT+CGPS=1,1 → enables GPS engine; AT+CGPSINFO → lat/lon/speed/time

- AT+CPWROFF → minimum power mode when cellular not needed

- HTTP POST: AT+HTTP* stack with SSL. Run HTTPTERM + CCHSTOP + NETCLOSE teardown before init. TLS 1.2 + SNI required for HTTPS (Google Apps Script). AT+SAPBR does NOT exist on this firmware.

⚠  T-Mobile SIM: disable SIM protection before deploying — confirmed registration blocker during bench testing.

## 2.3 Optocoupler Channel Assignment

| Opto Ch | 12V Signal Source | Meaning (active-LOW) | Pico GPIO | → RUN pin |
| --- | --- | --- | --- | --- |
| Ch 1 | Bilge pump #1 run line (mid) | LOW = mid bilge pump running | GP2 | YES |
| Ch 2 | Bilge pump #2 run line (stern) | LOW = stern bilge pump running | GP3 | YES |
| Ch 3 | Backup float switch #1 (mid) | LOW = water present in mid bilge | GP6 | YES |
| Ch 4 | Backup float switch #2 (stern) | LOW = water present in stern bilge | GP7 | YES |
| Ch 5 | Battery switch line | LOW = main battery switch ON | GP10 | No |
| Ch 6 | Reserved | — | GP11 | No |
| Ch 7 | Reserved | — | GP8 | No |
| Ch 8 | Reserved | — | GP9 | No |

⚠  Output VCC on all channels powered from Pico 3V3. Jumpers set to pull-up mode. All signals active-LOW. Ch1–4 output lines share a node via 10kΩ resistor to Pico RUN pin.

## 2.4 Full GPIO Pin Map

| GPIO | Protocol | Connected To | Notes |
| --- | --- | --- | --- |
| GP2 | Digital IN / IRQ | Opto Ch1 — Bilge pump #1 (mid) | LOW = running. Also → RUN pin. |
| GP3 | Digital IN / IRQ | Opto Ch2 — Bilge pump #2 (stern) | LOW = running. Also → RUN pin. |
| GP4 (TX) | UART1 | SIM7600X RXD breakout pin | Pico transmits → modem receives |
| GP5 (RX) | UART1 | SIM7600X TXD breakout pin | Modem transmits → Pico receives |
| GP6 | Digital IN / IRQ | Opto Ch3 — Float switch #1 (mid) | LOW = water present. Also → RUN pin. |
| GP7 | Digital IN / IRQ | Opto Ch4 — Float switch #2 (stern) | LOW = water present. Also → RUN pin. |
| GP8 (SDA) | I2C0 | INA260 #1 + INA260 #2 + INA219 (shared bus) | Different I2C addresses |
| GP9 (SCL) | I2C0 | INA260 #1 + INA260 #2 + INA219 (shared bus) |  |
| GP10 | Digital IN | Opto Ch5 — Battery switch signal | LOW = switch ON |
| GP14 | Digital OUT | SIM7600X RESET pin | Pull LOW to hard-reset modem if hung |
| GP20 | Digital OUT | Relay #1 coil | Reserved |
| GP21 | Digital OUT | Relay #2 coil | Reserved |
| RUN pin | Hardware reset | Shared node from Opto Ch1–4 via 10kΩ | Bilge wake from deep sleep |
| GP0/GP1 | UART0 | Reserved for USB serial debug only | Do NOT use for SIM7600X |

# 3. Operating Modes

## 3.1 Mode Detection Logic

| Pri. | Mode | Detection Condition | Action |
| --- | --- | --- | --- |
| 1 | Underway | Battery voltage > 13.5V for 3 consecutive readings | GPS every 30 sec, log speed/heading, cell upload |
| 2 | At Anchor/Dock | 12V switch line LOW (active-LOW) AND voltage 12.0–13.5V | GPS every 30 sec, anchor watch, cell upload |
| 3 | End-of-Day Alert | Switch line LOW AND battery switch ON for > 30 min | Send Pushover alert, log status, await switch-off |
| 4 | Winter Storage | Switch line HIGH (switch OFF — all others fail) | Hourly sensor read, WiFi upload, cell module OFF |

⚠  Bilge monitoring runs continuously via GPIO interrupts (in-season) or RUN pin wake (winter) regardless of mode.

## 3.2 Mode 1: Winter Storage

Pico is in deep sleep (~0.5mA). RTC alarm wakes every hour. SIM7600X is powered but in minimum power mode (AT+CPWROFF) between transmissions.

### Night wake cycle (configurable, e.g. 9pm–6am):

- Wake from deep sleep (~2 seconds total)

- Check bilge GPIO pins — if LOW, handle bilge event

- No sensor reads, no upload — return to sleep immediately

### Day wake cycle (configurable, e.g. 6am–9pm):

- Wake from deep sleep

- Read Google Sheet Config tab for current thresholds

- Read INA260 #1 (house solar/battery), INA260 #2 (engine solar/battery), INA219 (V50 health)

- Connect WiFi → POST to Google Sheet Power_Log → disconnect WiFi

- If WiFi fails: wake SIM7600X from low-power mode → POST via cellular → issue AT+CPWROFF → sleep

- Return to deep sleep

### Power budget (winter):

- Pico deep sleep: ~0.5mA

- SIM7600X in AT+CPWROFF mode: ~1–2mA idle

- Combined average: ~2–3mA → V50 (13,400mAh) lasts full 7-month winter season

- V50 recharges when battery switch turned ON (spring commissioning)

## 3.3 Mode 2: Underway (Engine Running)

- Detected when alternator charging pushes battery above 13.5V (INA260 #2)

- GPS every 30 sec via AT+CGPSINFO — log lat, lon, speed, heading

- Battery voltage logged every 5 min

- Cellular upload continuous

- Mode exits when voltage drops below 13.5V for 3 consecutive readings

## 3.4 Mode 3: At Anchor or Dock

- Anchor watch: capture GPS position on mode entry

- If boat drifts > 50m from anchor position → Pushover alert

- GPS every 30 sec, cellular upload

- Drift radius configurable via Google Sheet Config tab

## 3.5 Mode 4: End-of-Day Alert

- Trigger: battery switch line LOW (switch ON) for > 30 consecutive minutes

- Send Pushover: '⚠ Boat battery switch may be left on'

- Continue logging every 5 min

- When switch turns OFF: log event, power down cellular module

# 4. Bilge Pump Monitoring

## 4.1 Unified Wiring, Two Behaviors

The bilge optocoupler output is wired to BOTH a GPIO pin AND the Pico RUN pin simultaneously. One wiring works in all modes — no rewiring between seasons.

| Pico State | RUN Pin Behavior | GPIO Behavior |
| --- | --- | --- |
| Deep sleep (winter) | Pulse LOW → hard reboot. Code checks boot reason flag in flash, detects bilge event. | Read on startup to confirm which pump fired. |
| Always-on (in-season) | Brief LOW pulse — ignored by running firmware. | Hardware IRQ fires on RISING/FALLING edge. |

### RUN pin wiring:

- Opto Ch1–4 output lines → shared node → 10kΩ series resistor → Pico RUN pin

- Any bilge/float signal fires → shared node goes LOW → RUN pin goes LOW → Pico wakes

- On boot: read GP2, GP3, GP6, GP7 to identify which signal fired

- Boot reason flag written to flash before sleep: 'rtc' = normal wake, absent/stale = bilge wake

## 4.2 Bilge Event Logging

| Event | Trigger | Logged Fields | Alert? |
| --- | --- | --- | --- |
| Pump start | GP2 or GP3 LOW | Timestamp, pump_id (mid/stern), trigger | No |
| Pump stop | GP2 or GP3 HIGH | Timestamp, duration (sec) | If duration > 60 sec |
| High water — float | GP6 or GP7 LOW | Timestamp, location (mid/stern) | Yes — immediate Pushover |
| Repeated cycling | > 5 pump starts per hour | Count, timestamps | Yes |

⚠  In winter storage, if a bilge alert fires the Pico wakes the cellular module, sends Pushover alert, logs to Google Sheet, then powers the module back down and returns to sleep.

# 5. Connectivity & Data Flow

## 5.1 WiFi vs Cellular

| Mode | Primary | Fallback | SIM7600X state |
| --- | --- | --- | --- |
| Winter Storage | WiFi | Cellular | OFF — Pico in deep sleep. Wakes only for bilge alerts or WiFi failure. |
| Underway | Cellular | Local log | ON continuously |
| At Anchor/Dock | Cellular | WiFi if marina | ON continuously |
| End-of-Day Alert | Cellular | WiFi | ON until switch-off confirmed |

## 5.2 Google Sheets Structure

- Power_Log — timestamp, mode, house_solar_V, house_solar_A, engine_solar_V, engine_solar_A, usb_bank_V, charging

- GPS_Log — timestamp, mode, lat, lon, speed_kts, heading, fix_quality, distance_from_anchor

- Bilge_Log — timestamp, pump_id, trigger_type, duration_sec, float_active

- Events — timestamp, event_type, detail (mode changes, alerts, power transitions)

- Config — operational thresholds read by Pico on each wake cycle

✓  Single Apps Script endpoint accepts a 'tab' parameter — all data POSTs to the same URL with different tab names.

## 5.3 Configuration Split

| Setting | Location | Why |
| --- | --- | --- |
| WiFi SSID + password | config.py (BLE readable) | Sensitive — never in cloud |
| Pushover API key + device token | config.py (BLE readable) | Sensitive — never in cloud |
| T-Mobile APN (fast.t-mobile.com) | config.py | Rarely changes |
| GPIO pin assignments | config.py | Hardware-level |
| I2C addresses | config.py | Hardware-level |
| Winter day window (start/end hour) | Google Sheet Config tab | Tune seasonally |
| Poll interval (minutes) | Google Sheet Config tab | Tune anytime |
| Solar threshold (amps) | Google Sheet Config tab | Tune anytime |
| Low battery voltage alert threshold | Google Sheet Config tab | Tune anytime |
| Bilge alert duration (seconds) | Google Sheet Config tab | Tune anytime |
| Anchor watch radius (meters) | Google Sheet Config tab | Tune anytime |
| End-of-day alert timer (minutes) | Google Sheet Config tab | Tune anytime |

# 6. Power Chain

## 6.1 Power Topology

The Pico and SIM7600X share one 5V rail with common ground. Both are loads in parallel — never power the SIM7600X from the Pico's 3.3V or VSYS pin.

### Path A — Battery switch ON (PlusRoc 12V→5V):

- 12V battery switch → PlusRoc buck converter → 5V rail

- 5V rail → TPS2113A IN2 → Pico VSYS

- 5V rail → SIM7600X 5V power input (in parallel)

- 5V rail → V50 USB-C charging input (charges bank while switch is on)

- Common GND: battery negative, PlusRoc GND, Pico GND, SIM7600X GND

### Path B — Battery switch OFF (V50 always-on):

- V50 USB-A port → TPS2113A IN1 → Pico VSYS

- V50 second USB-A port → SIM7600X 5V power input

- TPS2113A autoswitches to IN1 when IN2 drops — seamless, no Pico power glitch

### SIM7600X power management (no MOSFET needed):

- Module auto-boots when 5V applied (PWR–3V3 jumper = default)

- Use AT+CPWROFF to put module in minimum power state between transmissions

- In winter deep sleep: issue AT+CPWROFF before Pico sleeps → module idles at ~1-2mA

- On wake: module auto-responds to AT commands after brief init, no PWRKEY pulse needed

⚠  PlusRoc buck converter is rated for ≥3A output — sufficient for Pico (~100mA) + SIM7600X LTE peak (~2A) with margin. Both loads share the same output rail.

## 6.2 Full Power Path Summary

### Path A — Always-on (V50 USB bank):

- V50 USB-A port #1 → USB-A to bare wire pigtail → TPS2113A IN1 (+) and GND

- V50 USB-A port #2 → SIM7600X 5V input

- Always live regardless of battery switch state

### Path B — Battery switch power:

- Boat battery switch 12V output → splits:

  - Branch 1 → PlusRoc buck converter INPUT (12V in → 5V out)

  - Branch 2 → Optocoupler board Ch5 INPUT (switch detection signal to GP10)

- PlusRoc 5V OUTPUT → splits:

  - Branch 1 → TPS2113A IN2 (+) — powers Pico when switch is on

  - Branch 2 → V50 USB-C charging input — charges V50 while switch is on

### TPS2113A output:

- TPS2113A OUT (+) → Pico VSYS pin (Pin 39)

- TPS2113A GND → Pico GND

# 7. External Wiring Harness

All external wires that connect the boat's systems to the monitoring box. Use this as the definitive reference when building the pigtail harness.

## 7.1 Bilge & Float Switch — 4 Wires

| # | Label | From (on boat) | To (in box) | Opto Ch | GPIO | → RUN |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | BILGE-MID-PUMP | Mid bilge pump run 12V+ | Opto Ch1 IN+ | Ch 1 | GP2 | YES |
| 2 | BILGE-STN-PUMP | Stern bilge pump run 12V+ | Opto Ch2 IN+ | Ch 2 | GP3 | YES |
| 3 | FLOAT-MID | Mid float switch 12V+ | Opto Ch3 IN+ | Ch 3 | GP6 | YES |
| 4 | FLOAT-STN | Stern float switch 12V+ | Opto Ch4 IN+ | Ch 4 | GP7 | YES |
| GND-A | BILGE-GND | Boat ground (bilge area) | Opto IN GND (Ch1–4) | — | — | — |

## 7.2 Battery Switch Signal — 1 Wire (splits inside box)

| # | Label | From (on boat) | To (in box) | GPIO |
| --- | --- | --- | --- | --- |
| 5 | BATT-SW-12V | Battery switch 12V output+ | SPLIT: Branch A → PlusRoc IN+ AND Branch B → Opto Ch5 IN+ | GP10 |
| GND-B | BATT-SW-GND | Battery switch ground | PlusRoc GND + Opto IN GND Ch5 | — |

## 7.3 Solar & Battery Monitoring — 4 Wires (INA260 in series)

| # | Label | From (on boat) | Connection | Measures |
| --- | --- | --- | --- | --- |
| 6 | SOLAR-HSE+ | House solar charger OUT+ (charger side of break) | INA260 #1 IN+ | House solar A + battery V |
| 7 | BATT-HSE+ | House battery+ (battery side of break) | INA260 #1 OUT+ |  |
| GND-C | HSE-GND | House battery/solar GND | INA260 #1 GND |  |
| 8 | SOLAR-ENG+ | Engine solar charger OUT+ (charger side of break) | INA260 #2 IN+ | Engine solar A + battery V |
| 9 | BATT-ENG+ | Engine battery+ (battery side of break) | INA260 #2 OUT+ | >13.5V = engine running |
| GND-D | ENG-GND | Engine battery/solar GND | INA260 #2 GND |  |

⚠  Break the solar charger + output wire and insert the INA260 in that break. Do NOT cut the battery − wire. INA219 for V50 bank is an internal box connection only.

## 7.4 INA260 I2C Addresses

- INA260 #1 (house): A0=GND, A1=GND → address 0x40

- INA260 #2 (engine): A0=VS, A1=GND → address 0x41

- INA219 (V50 bank): adjust A0/A1 to avoid conflict → address 0x44

- All three share GP4 (SDA) and GP5 (SCL)

## 7.5 Wire Gauge & Connectors

- Bilge/float signals (wires 1–4): 22 AWG — signal only

- Battery switch signal (wire 5): 18 AWG — carries up to 3A to PlusRoc

- Solar/battery monitoring (wires 6–9): match existing solar wiring gauge — INA260 is in the main current path

- Recommended connector: Deutsch DT series or Anderson Powerpole for power, JST for signal wires

- Label every wire at BOTH ends before installing

# 8. iOS App Screens

New screens added to the existing ballast monitor React Native / Expo app. Data source is Google Sheets read via Apps Script JSON endpoint.

| Screen | Content | Data Source |
| --- | --- | --- |
| Boat Status | Current mode badge, battery switch ON/OFF, last update time, quick-glance health summary | Events tab (latest row) |
| Power Dashboard | House + engine battery voltage gauges, solar current bars, V50 bank indicator, charging status | Power_Log tab |
| Location / Map | Live map pin, track line when underway, anchor marker, drift radius circle | GPS_Log tab |
| Bilge Status | Last pump run (pump ID, time, duration), float switch status, run count today, alert history | Bilge_Log tab |
| Settings | Alert thresholds, anchor radius, OTA firmware check, BLE connect for config update | Config tab + BLE |

# 9. Development Phases

| Phase | Goal | Tasks |
| --- | --- | --- |
| 1 ✓ | SIM7600X bench test | COMPLETE. Module confirmed on T-Mobile LTE Band 4 in Seattle. SIM PIN protection disabled. AT commands working. GPS engine confirmed. HTTP POST method being finalized. |
| 2 | HTTP POST to Google Sheets | Confirm AT+HTTP* stack with SSL (TLS 1.2 + SNI + HTTPTERM/CCHSTOP/NETCLOSE teardown). Test POST to Apps Script Web App endpoint. Confirm 200 response. |
| 3 | GPS confirmed | Test AT+CGPSINFO outdoors. Confirm lat/lon/speed output. Log to GPS_Log tab via HTTP POST. |
| 4 | INA sensor bench test | Wire INA260×2 + INA219 on shared I2C bus to Pico. Set unique addresses. Read voltage/current from 12V source. |
| 5 | Bilge interrupts | Wire optocoupler Ch1–4 to GP2/3/6/7 and shared RUN pin node. Simulate 12V pulses. Verify IRQ fires correctly. Test RUN pin wake from deep sleep. |
| 6 | Winter storage mode | Deep sleep + RTC wake. Day/night logic from Config tab. Read INAs. POST to Google Sheet via WiFi, cellular fallback. Boot reason flag in flash. |
| 7 | Mode detection + GPS tracking | Voltage-based mode switching with hysteresis. GPS every 30 sec. Anchor watch drift alert. End-of-day 30-min alert. |
| 8 | iOS app screens | Add Boat Status, Power Dashboard, Location/Map, Bilge Status, Settings to existing ballast app. |
| 9 | Field install + tuning | Install on boat. Verify all signals on real hardware. Tune thresholds. Enable hardware watchdog. |

# 10. Open Items

### Immediately pending:

- Confirm HTTP POST method to Google Sheets via SIM7600X (AT+HTTP* with SSL teardown — in progress via Cursor)

- GPS bench test outdoors — confirm AT+CGPSINFO returns valid fix

### Hardware verification needed:

- Confirm INA260 #1 and #2 I2C addresses — set A0/A1 solder pads to 0x40 and 0x41

- Confirm INA219 I2C address adjusted to 0x44 to avoid conflict

- RUN pin wiring: verify PC817 optocoupler outputs are open-collector. Wire shared node → 10kΩ series resistor → Pico RUN pin

- TPS2113A switching threshold: confirm IN2 (PlusRoc 5V) is preferred when battery switch is on

### Configuration decisions (set before winter deployment):

- Winter day window default: 6am–9pm (adjust for Seattle winter sunrise/sunset)

- End-of-day alert threshold: 30 min (tune after field test)

- Anchor watch radius: 50m (adjust based on anchorage conditions)

- Bilge alert threshold: 60 sec continuous run, 5 cycles/hour