Boat Monitor P2 — Wiring (Minimal Changes)

VSNS on pin 15 (done) · No more wire moves · New connections only

# SECTION 1 — ALREADY DONE (do not change)

| Pin | Goes to |
| --- | --- |
| 1, 2 | Engine INA260 SDA, SCL |
| 4, 5 | House INA260 SDA, SCL |
| 6, 7 | INA219 SDA, SCL |
| 9 | TPS STAT |
| 15 | TPS VSNS (you moved here — keep) |
| 17, 19, 20 | Red, Blue, Green LED |
| 39 | TPS OUT → VSYS |

Firmware: Pin(11) for VSNS. Do NOT change I2C or A0/A1. No more unsoldering.

INA260 terminals: VIN+ = battery/source side. VIN− = load side (not “OUT”).

# SECTION 2 — STILL TO DO

## SIM7600 (jumpers: UART=B, PWR=PWR-3V3, VCCIO=3.3V)

| Modem | Connect to |
| --- | --- |
| 5V | 5V rail |
| GND | GND rail |
| RXD | Pico pin 11 |
| TXD | Pico pin 12 |
| RST | Pico pin 14 (same side as UART) |

## Optocoupler — INPUT side (12V boat)

| Opto IN | Boat wire |
| --- | --- |
| Ch1 IN+ | Mid bildge |
| Ch2 IN+ | Aft bilgde |
| Ch3 IN+ | Mid water return |
| Ch4 IN+ | Aft water return |
| Ch5 IN+ | Switch (also → PlusRoc IN+) |
| Ch6 IN+ | Key |
| All IN− | Ground bus |

## Optocoupler — OUTPUT side (3.3V Pico)

| Opto OUT | Pico pin | Signal | RUN? |
| --- | --- | --- | --- |
| OUT1 | 31 | Mid bilge pump | Yes |
| OUT2 | 29 | Aft bilge pump | Yes |
| OUT3 | 25 | Mid water float | Yes |
| OUT4 | 24 | Aft water float | Yes |
| OUT5 | 26 | Switch | No |
| OUT6 | 27 | Key | No |
| OUT7-8 | 21, 22 | Spare | No |

GND: common bus. RUN: OUT1-4 → 10kΩ → pin 30.

## PlusRoc + V50 USB-C charge

| PlusRoc / V50 | Connect to |
| --- | --- |
| IN+ | Switch boat wire |
| OUT+ 5V | TPS IN2 + V50 USB-C charge IN (+) |
| Modem 5V | PlusRoc OUT+ and V50 USB-A #2 (+) each via diode |
| GND | Ground bus (all V50 −, PlusRoc IN−/OUT−) |

V50 USB-C = charge IN only (separate from USB-A #1 Pico and #2 modem).

## Switch vs Key

Switch = master battery ON (powers boat). Key = ignition ON even if engine not charging.

## Checklist

| ☐ | Task |
| --- | --- |
| ☐ | Modem 5V GND pins 11 12 14 |
| ☐ | Opto + RUN 10kΩ |
| ☐ | PlusRoc |
| ☐ | 16 boat wires (page 2) |

# SECTION 3 — Final pin map

| Pin | Status | Goes to |
| --- | --- | --- |
| 1-7, 9, 15, 17-20, 39 | DONE | I2C, STAT, VSNS, LEDs, VSYS |
| 11-12, 14 | TODO | Modem |
| 21-22 | spare | Opto Ch7-8 |
| 24-27, 29, 31 | TODO | Opto OUT1-6 |
| 30 | TODO | RUN 10k from OUT1-4 |

# SECTION 4 — 16 boat wires

| # | Label | Connect to |
| --- | --- | --- |
| 1 | Engine + | Bottom INA260 VIN+ |
| 2 | Engine − | Bottom GND + ground bus |
| 3 | House + | Middle INA260 VIN+ + both float 12V wires |
| 4 | House − | Middle GND + ground bus |
| 5 | Mid bildge | Opto Ch1 IN+ |
| 6 | Aft bilgde | Opto Ch2 IN+ |
| 7 | Key | Opto Ch6 IN+ |
| 8 | Switch | PlusRoc IN+ + Opto Ch5 IN+ |
| 9 | Mid water | House + (same as wire 3) |
| 10 | Mid water | Opto Ch3 IN+ |
| 11 | Solar engin | Bottom INA260 VIN+ |
| 12 | Solar engin | Bottom INA260 VIN− |
| 13 | Solar hous | Middle INA260 VIN+ |
| 14 | Solar hous | Middle INA260 VIN− |
| 15 | Aft water | House + (same as wire 3) |
| 16 | Aft water | Opto Ch4 IN+ |